Office Network Setup

Network Documentation Report

Cisco Packet Tracer Lab

Date: June 8, 2026

# 1. Network Overview

This document describes the office network implemented in Cisco Packet Tracer. The network follows a hierarchical three-tier model comprising a core layer (CORESW), a distribution layer (DISTR), and access layer switches (SW1, SW2). Internet connectivity is provided through Router0 using NAT/PAT, with OSPF as the dynamic routing protocol between the router and the core switch.

Key design features:

- Four VLANs: VLAN 10 (Data), VLAN 20 (Data), VLAN 99 (Management), VLAN 200 (Wireless)

- Centralised DHCP and DNS servers on VLAN 99

- Wireless infrastructure with WLC and two Lightweight Access Points

- Internet access simulated via DSL Modem, Cloud, and ISP-side servers (Google, YouTube)

- NAT overload on Router0 translates all 192.168.0.0/16 traffic to the WAN interface

# 2. Device Inventory

The following table lists all network devices in the topology.

| Hostname | Device Type | Role | IOS Version | Management IP |
| --- | --- | --- | --- | --- |
| Router0 | Router (PT) | Internet Gateway / NAT | 15.4 | 200.165.200.226 |
| CORESW | L3 Switch (3560) | Core Routing / Inter-VLAN | 12.2(37)SE1 | 192.168.99.1 |
| DISTR | L2 Switch | Distribution Switch | 15.0 | 192.168.99.201 |
| SW1 | L2 Switch | Access Switch 1 | 15.0 | 192.168.99.202 |
| SW2 | L2 Switch | Access Switch 2 | 15.0 | 192.168.99.203 |
| Switch0 | L2 Switch | ISP-side Switch | 15.0 | — |

# 3. VLAN Configuration

Four VLANs are configured across the network. Inter-VLAN routing is performed by the CORESW Layer 3 switch using Switched Virtual Interfaces (SVIs).

| VLAN ID | Name / Purpose | Gateway (CORESW SVI) | Notes |
| --- | --- | --- | --- |
| 10 | User / Data VLAN | 192.168.10.1 / 24 | PCs: PC0, PC1, PC3; DHCP helper → 192.168.99.100 |
| 20 | User / Data VLAN | 192.168.20.1 / 24 | PCs: PC2, PC4; DHCP helper → 192.168.99.100 |
| 99 | Management VLAN | 192.168.99.1 / 24 | All switch SVIs, DHCP Server, DNS Server, WLC |
| 200 | Wireless / Guest | 192.168.200.1 / 24 | Wireless clients via LAPs; DHCP helper → 192.168.99.100 |

# 4. IP Addressing Table

The table below covers all assigned IP addresses across infrastructure devices and end hosts.

| Device | Interface | IP Address | Subnet Mask | Gateway |
| --- | --- | --- | --- | --- |
| Router0 | Gi0/0/0 (inside) | 200.165.200.226 | 255.255.255.252 | — |
| Router0 | Gi0/0/1 (outside) | 10.0.0.100 | 255.0.0.0 | — |
| CORESW | Fa0/2 (routed) | 200.165.200.225 | 255.255.255.252 | 200.165.200.226 |
| CORESW | Vlan10 SVI | 192.168.10.1 | 255.255.255.0 | — |
| CORESW | Vlan20 SVI | 192.168.20.1 | 255.255.255.0 | — |
| CORESW | Vlan99 SVI | 192.168.99.1 | 255.255.255.0 | — |
| CORESW | Vlan200 SVI | 192.168.200.1 | 255.255.255.0 | — |
| DISTR | Vlan99 SVI | 192.168.99.201 | 255.255.255.0 | 192.168.99.1 |
| SW1 | Vlan99 SVI | 192.168.99.202 | 255.255.255.0 | 192.168.99.1 |
| SW2 | Vlan99 SVI | 192.168.99.203 | 255.255.255.0 | 192.168.99.1 |
| PC0 | Fa0 | 192.168.10.100 | 255.255.255.0 | 192.168.10.1 |
| PC1 | Fa0 | 192.168.10.101 | 255.255.255.0 | 192.168.10.1 |
| PC2 | Fa0 | 192.168.20.103 | 255.255.255.0 | 192.168.20.1 |
| PC3 | Fa0 | 192.168.20.100 | 255.255.255.0 | 192.168.20.1 |
| PC4 | Fa0 | 192.168.10.104 | 255.255.255.0 | 192.168.10.1 |
| DHCP Server | Fa0 | 192.168.99.100 | 255.255.255.0 | 192.168.99.1 |
| DNS Server | Fa0 | 192.168.99.102 | 255.255.255.0 | 192.168.99.1 |
| WLC | — | 192.168.99.101 | 255.255.255.0 | 192.168.99.1 |
| Google Server | Fa0 | 10.0.0.120 | 255.0.0.0 | — |
| YouTube Server | Fa0 | 10.0.0.130 | 255.0.0.0 | — |

# 5. Port Configuration

## 5.1 Trunk Links

Trunk ports carry traffic for all VLANs between distribution and access switches.

| Switch | Port | Connected To | Mode |
| --- | --- | --- | --- |
| DISTR | Fa0/1 | CORESW | Trunk |
| DISTR | Fa0/2 | SW1 | Trunk |
| DISTR | Fa0/3 | SW2 | Trunk |

## 5.2 Access Ports

Access ports connect end devices to a specific VLAN on SW1 and SW2.

| Switch | Port | VLAN | Connected Device | Mode |
| --- | --- | --- | --- | --- |
| SW1 | Fa0/2 | 10 | PC0 | Access |
| SW1 | Fa0/3 | 10 | PC1 | Access |
| SW1 | Fa0/4 | 20 | PC2 | Access |
| SW1 | Fa0/5 | 10 | DHCP PC6 | Access |
| SW1 | Fa0/6 | 200 | Light Weight AP0 | Access |
| SW2 | Fa0/2 | 20 | PC3 | Access |
| SW2 | Fa0/3 | 10 | PC4 | Access |
| SW2 | Fa0/4 | 20 | — | Access |
| SW2 | Fa0/5 | 200 | Smartphone0 | Access |
| SW2 | Fa0/6 | 200 | Light Weight AP2 | Access |

# 6. Routing Configuration

OSPF process 1 in Area 0 is used for dynamic route exchange between Router0 and CORESW. Static default routes supplement OSPF for internet-bound traffic.

| Device | Protocol | Network | Notes |
| --- | --- | --- | --- |
| Router0 | OSPF (Area 0) | 192.168.0.0/16, 10.0.0.0/8, 200.165.200.0/24 | NAT overload on Gi0/0/0 |
| Router0 | Static | 192.168.0.0/16 | Next-hop 200.165.200.225 (CORESW) |
| CORESW | OSPF (Area 0) | 192.168.0.0/16, 10.0.0.0/8, 200.165.200.0/24 | ip routing enabled; inter-VLAN routing |
| CORESW | Default Static | 0.0.0.0/0 | Next-hop 200.165.200.226 (Router0) |

# 7. Network Services

| Service | Device / IP | Details |
| --- | --- | --- |
| DHCP | 192.168.99.100 | Centralized DHCP server; CORESW SVIs use ip helper-address for VLANs 10, 20, 200 |
| DNS | 192.168.99.102 | Internal DNS server on VLAN 99 |
| WLC (Wireless) | 192.168.99.101 | Manages Light Weight APs (LAP0, LAP2); wireless clients on VLAN 200 |
| NAT (PAT) | Router0 | Access-list 1 permits 192.168.0.0/16; overloaded on Gi0/0/0 |
| OSPF | Router0 + CORESW | Process 1, Area 0; advertises all internal and ISP-link networks |
| STP | All switches | PVST mode on all switches |
| VTY Access | DISTR, SW1, SW2 | Password: cisco (plaintext — not encrypted) |

# 8. Topology Description

The network topology follows a hierarchical design as observed in the Packet Tracer screenshot:

- Internet Side: Router0 connects to the ISP cloud via DSL Modem0 (Gi0/0/1 → 10.0.0.100). Servers simulating Google (10.0.0.120) and YouTube (10.0.0.130) reside on Switch0 connected to the ISP cloud.

- WAN Link: Router0 Gi0/0/0 (200.165.200.226) connects to CORESW Fa0/2 (200.165.200.225) via a /30 point-to-point link.

- Core Layer: CORESW acts as the Layer 3 routing hub. It hosts SVIs for all four VLANs and connects to DHCP Server, DNS Server, and WLC via VLAN 99 access ports (Fa0/3, Fa0/4, Fa0/5).

- Distribution Layer: DISTR switch connects to CORESW (Fa0/1 trunk) and fans out to SW1 (Fa0/2 trunk) and SW2 (Fa0/3 trunk).

- Access Layer: SW1 and SW2 connect end-user PCs and Lightweight Access Points. Wireless clients (Tablet PC0, Smartphone0) associate through the APs managed by the WLC.

# 9. Security Notes

- VTY passwords on DISTR, SW1, and SW2 are set to 'cisco' in plaintext. In a production environment, service password-encryption and stronger credentials should be applied.

- CORESW and Router0 VTY lines have no password configured — access control should be added.

- NAT ACL (access-list 1) correctly limits translation to 192.168.0.0/16 internal addresses.

- VLAN 1 is shut down on all switches — best practice followed.