OSPF Routing Lab — Network Documentation

# 1. Network Topology

Three routers (Router3, Router4, Router5) are connected in a chain via serial links. Router3 also has a loopback interface. All routers run OSPF in Area 0.

| Router | Interface | IP Address | Connected To | OSPF Process |
| --- | --- | --- | --- | --- |
| Router3 (R1) | Serial2/0 (Se2/0) | 10.0.1.1/8 | Router4 Serial2/0 | OSPF 1 |
| Router3 (R1) | Loopback1 | 9.0.0.9/8 | Loopback (local) | OSPF 1 |
| Router4 (R2) | Serial2/0 (Se2/0) | 10.0.1.2/8 | Router3 Serial2/0 | OSPF 2 |
| Router4 (R2) | Serial3/0 (Se3/0) | 11.0.2.1/8 | Router5 Serial2/0 | OSPF 2 |
| Router5 (R3) | Serial2/0 (Se2/0) | 11.0.2.2/8 | Router4 Serial3/0 | OSPF 3 |

# 2. OSPF Configuration

Each router runs a separate OSPF process (locally significant). All interfaces are advertised into Area 0 using wildcard masks.

| Router | Process ID | Router ID | Network Statement 1 | Network Statement 2 |
| --- | --- | --- | --- | --- |
| Router3 (R1) | 1 | 10.0.1.1 | 10.0.0.0 0.255.255.255 area 0 | 9.0.0.0 0.255.255.255 area 0 |
| Router4 (R2) | 2 | 11.0.2.1 | 10.0.0.0 0.255.255.255 area 0 | 11.0.0.0 0.255.255.255 area 0 |
| Router5 (R3) | 3 | 11.0.2.2 | 11.0.0.0 0.255.255.255 area 0 | — |

# 3. Routing Tables (show ip route)

'C' = directly connected, 'O' = learned via OSPF. Metrics shown as [Administrative Distance / Cost].

| Router | Code | Network | Next Hop / Interface | Metric [AD/Cost] |
| --- | --- | --- | --- | --- |
| Router3 (R1) | C | 9.0.0.0/8 | Loopback1 (direct) | directly connected |
| Router3 (R1) | C | 10.0.0.0/8 | Serial2/0 (direct) | directly connected |
| Router3 (R1) | O | 11.0.0.0/8 | 10.0.1.2 via Serial2/0 | [110/128] |
| Router4 (R2) | O | 9.0.0.0/32 | 10.0.1.1 via Serial2/0 | [110/65] |
| Router4 (R2) | C | 10.0.0.0/8 | Serial2/0 (direct) | directly connected |
| Router4 (R2) | C | 11.0.0.0/8 | Serial3/0 (direct) | directly connected |
| Router5 (R3) | O | 9.0.0.0/32 | 11.0.2.1 via Serial2/0 | [110/129] |
| Router5 (R3) | O | 10.0.0.0/8 | 11.0.2.1 via Serial2/0 | [110/128] |
| Router5 (R3) | C | 11.0.0.0/8 | Serial2/0 (direct) | directly connected |

# 4. OSPF Neighbor Adjacencies (show ip ospf neighbor)

All adjacencies are in FULL state. Priority 0 indicates no DR/BDR election (point-to-point serial links).

| Router | Neighbor ID | Pri | State | Address | Interface |
| --- | --- | --- | --- | --- | --- |
| Router3 (R1) | 11.0.2.1 | 0 | FULL/- | 10.0.1.2 | Serial2/0 |
| Router4 (R2) | 9.0.0.9 (R1) | 0 | FULL/- | 10.0.1.1 | Serial2/0 |
| Router4 (R2) | 11.0.2.2 (R3) | 0 | FULL/- | 11.0.2.2 | Serial3/0 |
| Router5 (R3) | 11.0.2.1 (R2) | 0 | FULL/- | 11.0.2.1 | Serial2/0 |

# 5. Key Observations & Notes

| Item | Detail |
| --- | --- |
| OSPF Area | All routers participate in Area 0 (backbone area) |
| Different Process IDs | R1=1, R2=2, R3=3 — process IDs are locally significant; OSPF still forms adjacencies across different process IDs |
| Router ID Selection | R1: 10.0.1.1 (highest IP / loopback 1.1.1.1 / loopback 9.0.0.9). R2: 11.0.2.1. R3: 11.0.2.2 |
| Loopback on R1 | 9.0.0.9/8 loopback is advertised into OSPF and visible at R2 as a /32 subnetted route [110/65] |
| AD/Cost | OSPF Administrative Distance = 110. Cost is based on interface bandwidth (serial default = 64 cost per hop) |
| Serial Clock Rate | R1 Serial3/0 set to 2000000; R4 Serial2/0 set to 2000000; R5 Serial2/0 and Serial3/0 set to 2000000 (DCE side) |

# 6. Logical Topology Diagram

Router3 (R1)  —[Se2/0 10.0.1.1 ←→ 10.0.1.2 Se2/0]—  Router4 (R2)  —[Se3/0 11.0.2.1 ←→ 11.0.2.2 Se2/0]—  Router5 (R3)

Router3 also has Loopback1: 9.0.0.9/8 (advertised into OSPF Area 0).

Network Labels visible on topology :

Router3: Router ID 10.0.1.1 | 1.1.1.1 | 9.0.0.9

Router4: Router ID 11.0.2.1

Router5: Router ID 11.0.2.2